2000 Pontiac Sunfire Alarm, Remote Start, Keyless Entry Wiring Information. Constant 12V+; Red; Ignition Switch Harness. Starter; Yellow; Ignition Switch Harness. Ignition; Pink; Ignition Switch Harness. Ignition 2; White; Ignition Switch Harness PCM and Cluster. Accessory; Orange; Ignition Switch Harness (Heat/AC). Tach; White; Harness near Battery. Neutral Safety Wire; Yellow; Ignition Switch Harness. Brake Switch; White; Brake Switch. Trunk Release; Black/White (+); OEM Keyless Module Driver's Side of Trunk. Trunk Pin; Black (+); Light in Trunk. Parking Lights; Brown (+); Light Switch. Head Lamp; Dark Blue (-); Pink (-) at Head Light Switch. Door Trigger; Green/Black (-); And LT. Blue Block near Brake Diode Isolate. Door Lock; Light Blue; Either Kick Panel (Passenger's Switch is Master). Door Unlock; White; Reverse Polarity. Driver's Lock; Gray; Driver's Kick Panel. Driver's Unlock; Tan; Driver's Kick Panel. Horn Wire; Black (-); HEADLamp Switch. Windows Up; LF=Dark Blue, RF=Light Blue, LR=Dark Green, RR=Light Green. Windows Down; LF=Brown, RF=Tan, LR=Purple, RR=Purple..